Lab 1: Introduction to Verilog HDL

103061207 徐安廷

Design Specification

- Full Adder

- Logic Equation:  S=a⊕b⊕Ci ,  Co=a’b+aci+bci

- Logic Diagram:

- Block Diagram:

- I/O:

Half Adder:

Inputs: a, b.

Outputs: s, co.

Full Adder:

Inputs: a, b, ci.

Outputs: co, s.

- Single Digit Decimal Adder

- Block Diagram:

- I/O:

Inputs: a[3:0], b[3:0], ci.

Outputs: result[3:0], s[3:0], co, cout.

- 2-to-4-Line Decoder

- Logic Equation:  d[0]=E．in[1]’．in[0]’ ,  d[1]=E．in[1]’．in[0] ,

d[2]=E．in[1]．in[0]’ ,  d[3]=E．in[1]．in[0].

- Block Diagram:

- I/O:

Inputs: E, in[1:0].

Output: d[3:0].

Design Implementation

- Full Adder

- a and b is the bit we like to add, and ci is an one bit carry in.

s is the result, and co is an one bit carry out.

- The logic function should be:   s=a^b^Ci ,  co=a’b|ac|bc

- Single Digit Decimal Adder

- Logic function:

When adding input a and b, if the result is larger than 9, it should add 6 to the result. So, we will get a one bit carry out and a four-bit binary number result[3:0].

Cout=K+Z8Z4+Z8Z2

- 2-to-4-Line Decoder

- Logic function

E is enable. When E=0, the decoder will output 0000.

D[0] = E & (~in[1]) & (~in[0])

D[1] = E & (~in[1]) & (in[0])

D[2] = E & (in[1]) & (~in[0])

D[3] = E & (in[1]) & (in[0])

Discussion

- It was really a happy experiment, only when debugging made me disappointed. The only wrong thing a met is I forgot to link the full adders in the second 4-bit adder. So, it is not weird that the simulation turned out lots of red Xs. The experiment results went all correct, since I follow the references in the hangout, so it ran as I expected.

Conclusion

- I have got familiar with the use of Verilog, and know how to declare I/Os variables and write a testbench to test my circuit. During the debugging, I tried to understand the error messages. Hope it will help me to finish future projects.

References

- Teaching handouts <Introduction> p.52:

It helps making a decimal adder without using any if statements, but by only using the simple logic gates to get the goal.